CALIFORNIA OLIVES

This scenario is intended to represent olive production in California’s San Joaquin Valley, primarily in Tulare, Madera, Fresno, and Kings Counties, hereafter referred to as the Area of Interest (AOI).  Nearly 65% of California’s olive acreage is located in the AOI, primarily in Tulare County (53% of the total) (USDA, 1999).  Moreover, based on 2005 California Pesticide Usage Reporting (CalPIP, 2006), approximately 80% of pesticides used on California olives (by pounds of chemical) were applied in the AOI.  Lesser amounts of olives are grown and lesser amounts of pesticides are applied in the Sacramento Valley, primarily Butte, Glenn and Tehama counties (CalPIP, 2006).  Scenario parameters were selected to be generally representative of Tulare County.  The County is located on the extreme southern end of the Great San Joaquin Valley and is partially surrounded by a horseshoe-shaped rim of mountains (NOAA 2006).

Over 99% of the olives grown in the United States are grown in California with approximately 35,300 bearing acres of olives (USDA, 1999). The olive is a thick evergreen tree that is cultivated in California for use in the table olive industry. The table olive industry includes four commercially grown olive (Olea europea L.) cultivars (CVs): Manzanillo (the most common), Sevillano, Ascolano, and Mission which account for over 99% of the total olive acreage in the State (USDA, 1999). Olives grown in California cover approximately 33,051 acres in the San Joaquin Valley and Northern Sacramento Valley and yields approximately 4-7 tons/acre (University of California, 1995). The numbers of acreage in increasing aaccording to the USDA Crop Profile for Olives in California in which there are approximately 35,300 acres of olives (USDA, 1999) in 1999 and in 2002 the USDA Census of Agriculture estimated 38,783 acres of olives in California (USDA, 2002).

Olives can grow on a wide range of soils and soil quality with approximately 38-39 acre inches of water per acre per year required for optimal production in the San Joaquin Valley (USDA, 1999). Drip and micro-sprinkler irrigation are the most popular methods of irrigation, used by approximately 70% of the acreage (USDA, 1999). Other types of irrigation methods used include flood, furrow, sprinkler, drip and micro sprayer. The typical spacing of olive trees are: standard: 30' x 30' (48 trees/acre); high density: 30'x 30' hexagonal/equilateral triangle design (56 trees/acre); and hedgerow: 15' x 30' (97 trees/acre) (University of California, 1995).  Harvesting begins when 50% of the olives taken in daily samples fall within standard medium, large, or extra large size and the percentage is increasing at a rate of 3-5% per week, beginning in mid-September and finishing in mid-November (University of California, 1995).

The Porterville series is the third most common soil found in the AOI where olives are grown, accounting for 25% of olive bearing soils (USDA, 2006; Table 5).  It is a fine, smectitic, thermic Aridic Haploxererts soil found on slopes of 0 to 15% (USDA, 1997; USDA, 2006).  The soils are of moderate extent and are used mainly for range, pasture, irrigated oranges, lemons, olives, figs and some grapes (USDA, 1997).  A local extension agent also confirmed that olives are grown on Porterville soils in the AOI (B. Peacock, personal communication).  Location and metfile selections are often the most important developments affecting scenario vulnerability and protectiveness.  Because some olive production occurs in Northern California where it is generally wetter (NOAA, 2006), a comparison of soils in both Area I (Tulare, Madera, Fresno, and Kings Counties) and Area II (Butte, Glenn and Tehama Counties) was performed.  In Area I (primary olive growing area), nearly 65% of olive bearing soils are in hydrologic group D.  Conversely, nearly 97% of olive bearing soils in Area II are in hydrologic group C.  Because hydrologic group significantly impacts curve number selection, the most sensitive parameter in PRZM (hydrologic group D soils have a higher tendency for runoff), this scenario is based in the San Joaquin Valley due to the extensive hydrologic group D olive bearing soils in the area.  Based on USDA soils data (USDA, 2006), olive bearing soils in Area I (Table 5), are also located on steeper slopes than olive bearing soils in Area II (Table 6), which also increase runoff vulnerability (Table 5).

Porterville is a Hydrologic Group D soil, which includes nearly 65% percent of olive bearing soils in drainage.  Porterville soils have a USLE K factor of 0.24-0.37 and includes the 90th percentile of soils in erodibility. Less than 2% of soils in the AOI have a USLE K factor greater than 0.37 (Table 5).  70% of olive bearing soils in the AOI have a pH lower than Porterville soils (7.5).  However, soil pH is not currently a PRZM input parameter and is not expected to often affect chemical fate in the acidic range.  Porterville soils have an A horizon from 0 to 27 inches (0-69 cm) deep and a C horizon from 27 to 71 inches (69-180 cm) deep (USDA, 1997).  No benchmark soils of California were selected for this scenario because the benchmark soils identified (e.g., Exeter) was a less vulnerable soil for runoff based on hydrologic group C (USDA, 2006).

Two metfiles are located roughly within 65 miles of Tulare County.  Bakersfield (Metfile W23155) is located approximately 61 miles to the south of the center of Tulare County.  Fresno is located approximately 64 miles to the northwest of the center of Tulare County; however it is a more representative station of the AOI, which includes Fresno and Madera Counties.  Fresno was chosen since it receives approximately five inches more rainfall per year than Bakersfield (NOAA, 2006), making it a more conservative scenario.  The Met station (W93193.dvf) is located at 36º 47’ N, 119º 43’ W and at an elevation of approximately 102 meters above sea level.  This station receives an average of approximately 11 inches (28 cm) of rainfall annually, with the majority of rainfall occurring between November and March (NOAA, 2006.  Snow in the valley is infrequent, with only a trace occurring in about one year out of seven).

| Table 1. PRZM 3.12 Climate and Time Parameters for Tulare County, California – Olives. |  |  |
| --- | --- | --- |
| Parameter | Value | Source/Comments |
| Starting Date | Jan. 1, 1961 | Meteorological File from Fresno, California (W93193). |
| Ending Date | Dec. 31, 1990 | Meteorological File from Fresno, California (W93193). |
| Pan Evaporation Factor (PFAC) | 0.70 | PRZM Manual, Figure 5.1 (EPA, 1998). Value represents south-central California. |
| Snowmelt Factor (SFAC) | 0 | The Weather Channel Interactive, Inc. (TWCII, 2006) and PRZM Manual, Table 5.1 (EPA, 1998). |
| Minimum Depth of Evaporation (ANETD) | 32.5 cm | PRZM Manual, Figure 5.2 (EPA, 1998). Midpoint of the reported range for south-central California. |

| Table 2. PRZM 3.12 Erosion and Landscape Parameters for Tulare County, California – Olives. |  |  |
| --- | --- | --- |
| Parameter | Value | Source/Comments |
| Method to Calculate Erosion (ERFLAG) | 4 (MUSS) | Guidance for Selecting Field Crop and Orchard Scenario Input Parameters (EPA, 2004) |
| USLE K Factor (USLEK) | 0.37 tons EI-1* | USDA NRCS Soil Data Mart (http://soildatamart.nrcs.usda.gov/) Value listed for the soil series Porterville Approximate values are listed in PRZM Manual, Table 5-3 (EPA, 1998). |
| USLE LS Factor (USLELS) | 1.98 | PRZM Manual, Table 5-5 (EPA, 1998); LS equation (Haan and Barfield, 1978) LS value for 400’ slope length and 8% slope Bill Peacock, Farm Advisor, University of California Cooperative Extension |
| USLE P Factor (USLEP) | 1.0 | PRZM Manual, Table 5-6 (EPA, 1998) Default for olives with no contour practices. Bill Peacock, Farm Advisor, University of California Cooperative Extension |
| Field Area (AFIELD) | 172 ha | Area of Shipman Reservoir watershed (EPA, 1999) |
| NRCS Hyetograph (IREG) | 1 | PRZM Manual, Figure 5.12 (EPA, 1998) |
| Slope (SLP) | 8% | Set to the midpoint for the soil range (0-15%). This is a reasonable value, but slopes can be as high as 12% based according to Bill Peacock, Farm Advisor, University of California Cooperative Extension. Guidance for Selecting Field Crop and Orchard Scenario Input Parameters (EPA, 2004). |
| Hydraulic Length (HL) | 600 m | Shipman Reservoir (EPA, 1999) |
| Irrigation Flag (IRFLAG) | 1 | Drip and micro-sprinkler irrigation are the most popular methods of irrigation, used by approximately 70% of the acreage (USDA, 1999). Fogger (low volume) irrigation used on 80% of crops and the remainder (20%, high volume) is furrow irrigation. Bill Peacock, Farm Advisor, University of California Cooperative Extension |
| IRTYPE | 4 (drip) | Drip and micro-sprinkler irrigation are the most popular methods of irrigation, used by approximately 70% of the acreage (USDA, 1999). |
| Leaching Factor (FLEACH) | 0.0 | Irrigation Guidance for developing PRZM Scenarios, Table 3; (June 15, 2005). Default value for drip irrigation. |
| Fraction of Water Capacity when Irrigation is Applied (PCDEPL) | 0.50 | Set to default value. Irrigation Guidance for developing PRZM Scenario, Table 3; (EPA 2005). |
| Maximum Rate at which Irrigation is Applied (RATEAP) | 0.037 cm hr-1 | Irrigation Guidance for developing PRZM Scenarios, Table 1; (June 15, 2005). For CN = 85 and f = 0 |

| Table 3. PRZM 3.12 Crop Parameters for Tulare County, California – Olives. |  |  |
| --- | --- | --- |
| Parameter | Value | Source/Comments |
| Initial Crop (INICRP) | 1 | Set to one for all crops. Guidance for Selecting Field Crop and Orchard Scenario Input Parameters (EPA, 2004). |
| Initial Surface Condition (ISCOND) | 3 | Bill Peacock, Farm Advisor, University of California Cooperative Extension |
| Number of Different Crops (NDC) | 1 | Set to number of crops in simulation. |
| Number of Cropping Periods (NCPDS) | 30 | Set to weather data in meteorological file from Fresno, California (W93193). |
| Maximum rainfall interception storage of crop (CINTCP) | 0.25 | Recommended value for orchards. Guidance for Selecting Field Crop and Orchard Scenario Input Parameters (EPA, 2004). |
| Maximum Active Root Depth (AMXDR) | 60 cm | Bill Peacock, Farm Advisor, University of California Cooperative Extension |
| Maximum Canopy Coverage (COVMAX) | 90% | Bill Peacock, Farm Advisor, University of California Cooperative Extension |
| Soil Surface Condition After Harvest (ICNAH) | 3 | Bill Peacock, Farm Advisor, University of California Cooperative Extension |
| Date of Crop Emergence (EMD, EMM, IYREM) | 01/01 | Values are set to keep E/T and canopy coverage terms working correctly for this evergreen scenario. |
| Date of Crop Maturity (MAD, MAM, IYRMAT) | 02/01 | Values are set to keep E/T and canopy coverage terms working correctly for this evergreen scenario. |
| Date of Crop Harvest (HAD, HAM, IYRHAR) | 31/12 | Values are set to keep E/T and canopy coverage terms working correctly for this evergreen scenario. |
| Maximum Dry Weight (WFMAX) | 0 | Not used in scenario (EPA, 2004) |
| Maximum Canopy Height (HTMAX) | 457.2 cm | Bill Peacock, Farm Advisor, University of California Cooperative Extension |
| SCS Curve Number (CN) | 87, 85, 86 | Gleams Manual Table H-4; meadows; no fallow conditions (USDA, 1990) |
| Manning’s N Value (MNGN) | 0.070 | RUSLE Project; C23OFOFN for California (Fresno) orchards with full cover (USDA, 2000). These data are the closest data available to represent olives in Central CA. |
| USLE C Factor (USLEC) | 0.003 - 0.029 | RUSLE Project; C23OFOFN for California (Fresno) orchards with full cover (USDA, 2000). These data are the closest data available to represent olives in Central CA. |

| Table 4. PRZM 3.12 Porterville Clay, 9 To 15 Percent slopes Soil Parameters for Tulare County, California – Olives. |  |  |
| --- | --- | --- |
| Parameter | Value | Source/Comments |
| Total Soil Depth (CORED) | 285 cm | NRCS Official Soil Series Descriptions (OSD) (http://soils.usda.gov/technical/classification/osd/index.html) (USDA, 2006) |
| Number of Horizons (NHORIZ) | 4 | NRCS OSD (USDA, 2006); soil consists of two horizons: A and C. The A horizon spans scenario horizons 1 and 2 in order to conform to PRZM input requirements. |
| Horizon Thickness (THKNS) | 10 cm (HORIZN = 1) 71 cm (HORIZN = 2) 102 cm (HORIZN = 3) 102 cm (HORIZN = 4) | NRCS Soil Data Mart (SDM) (http://soildatamart.nrcs.usda.gov) and NRCS OSD |
| Bulk Density (BD) | 1.35 g/cm3 (HORIZN = 1) 1.35 g/cm3 (HORIZN = 2) 1.35 g/cm3 (HORIZN = 3) 1.35 g/cm3 (HORIZN = 4) | NRCS Soil Data Mart (SDM) (http://soildatamart.nrcs.usda.gov); values are mean 1/3-bar moist bulk densities. |
| Initial Water Content (THETO) | 0.366 cm3/cm3 (HORIZN =1) 0.366 cm3/cm3 (HORIZN =2) 0.395 cm3/cm3 (HORIZN =3) 0.395 cm3/cm3 (HORIZN =4) | NRCS Soil Data Mart (SDM) (http://soildatamart.nrcs.usda.gov); values are mean 1/3-bar water contents of a soil sampled as a Porterville soil. |
| Compartment Thickness (DPN) | 0.1 cm (HORIZN = 1) 1.0 cm (HORIZN = 2) 3.0 cm (HORIZN = 3) 3.0 cm (HORIZN = 4) | NRCS Soil Data Mart (SDM) (http://soildatamart.nrcs.usda.gov); Guidance for Selecting Field Crop and Orchard Scenario Input Parameters (EPA, 2004). |
| Field Capacity (THEFC) | 0.366 cm3/cm3 (HORIZN =1) 0.366 cm3/cm3 (HORIZN =2) 0.395 cm3/cm3 (HORIZN =3) 0.395 cm3/cm3 (HORIZN =4) | NRCS Soil Data Mart (SDM) (http://soildatamart.nrcs.usda.gov); values are mean 1/3-bar water contents of a soil sampled as a Porterville soil. |
| Wilting Point (THEWP) | 0.264 cm3/cm3 (HORIZN =1) 0.264 cm3/cm3 (HORIZN =2) 0.261 cm3/cm3 (HORIZN =3) 0.261 cm3/cm3 (HORIZN =4) | NRCS Soil Data Mart (SDM) (http://soildatamart.nrcs.usda.gov); values are mean 15-bar water contents of a soil sampled as a Porterville soil. |
| Organic Carbon Content (OC) | 1.16% (HORIZN = 1) 1.16% (HORIZN = 2) 0.44% (HORIZN = 3) 0.44% (HORIZN = 4) | NRCS SDM; values for horizons 1 to 4 = mean %OM / 1.724. |

| Table 5. Olive Bearing Soils of Tulare Counties Area, California – Ranked by Area. |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Soil | Total Acreage | % Area | Drainage | Erodibility | Slopes (%) | pH | OM (%) | % Sand | % Silt | Olives |  |
|  |  |  |  |  |  |  |  |  |  | Irrigated | Non-Irr |
| CENTERVILLE | 56,886 | 33.75% | D | 0.2-0.24 | 0-30 | 7.3-7.5 | 1.5 | 22-22.1 | 27.9-28 | 3 | - |
| EXETER | 46,249 | 27.44% | C | 0.24-0.37 | 0-9 | 6.7 | 0.5-1.5 | 44-66 | 19-41 | 4 | - |
| PORTERVILLE | 43,154 | 25.61% | D | 0.24-0.37 | 0-15 | 7.5 | 1.5-2 | 22-22.1 | 27.9-28 | 3-4 | - |
| HILDRETH | 5,100 | 3.03% | D | 0.24 | 0-5 | 7.2-7.5 | 2 | 22-49.8 | 7.7-28 | 2.5 | - |
| WYMAN | 4,892 | 2.90% | B | 0.37-0.43 | 0-5 | 6.7 | 1.5 | 43 | 39.5 | 4 | - |
| YETTEM | 4,833 | 2.87% | B | 0.24 | 0-2 | 6.7 | 1.5 | 66 | 22.5 | 4 | - |
| HONCUT | 2,952 | 1.75% | B | 0.32 | 0-2 | 6.7 | 1.5 | 67.3 | 23.2 | 4 | - |
| SEVILLE | 2,262 | 1.34% | D | 0.2 | 0-2 | 7.5 | 1.5 | 22.1 | 27.9 | 3 | - |
| RAYNOR | 1,269 | 0.75% | D | 0.24 | 0-8 | 7.5 | 1.5 | 22.1 | 27.9 | 3 | - |
| LEWIS | 940 | 0.56% | D | 0.43 | 0-1 | 8.5 | 0.75 | 39.2 | 37.3 | 1 | - |

| Table 6. Olive Bearing Soils of Tehema, Glenn, and Butte Counties Area, California – Ranked by Area. |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Soil | Total Acreage | % Area | Drainage | Erodibility | Slopes (%) | pH | OM (%) | % Sand | % Silt | % Clay | Olives |  |
|  |  |  |  |  |  |  |  |  |  |  | Irrigated | Non-Irr |
| TEHAMA | 62,581 | 96.62% | C | 0.37-0.43 | 0-5 | 6.1-6.3 | 0.5-1.25 | 11.4-68 | 16-70.1 | 16-20 | 2 | - |
| STOCKTON | 1,271 | 1.96% | D | 0.24 | 0-1 | 6.7 | 1.5 | 22.1 | 27.9 | 50 | 3 | - |
| PORTERVILLE | 919 | 1.42% | D | 0.28 | 0-10 | 6.3 | 2 | 22.1 | 27.9 | 50 | 4 | - |

Works Cited

CalPIP.  2006.  2005 Pesticide Usage Reporting.  California Pesticide Information Portal, CA Department of Pesticide Regulation.  Accessed December 4, 2006.  Online at: http://calpip.cdpr.ca.gov/cfdocs/calpip/prod/main.cfm.

EPA. 1998. Carsel, R.F., J.C. Imhoff, P.R. Hummel, J.M. Cheplick, and A.S. Donigian, Jr. PRZM-3, A Model for Predicting Pesticide and Nitrogen Fate in the Crop Root and Unsaturated Soil Zones: Users Manual for Release 3.0. National Exposure Research Laboratory, Office of Research and Development, U.S. Environmental Protection Agency, Athens, GA. Online at: http://www.epa.gov/ceampubl/gwater/przm3/przm312b.htm.

EPA. 1999. Jones, R.D., J. Breithaupt, J. Carleton, L. Libelo, J. Lin, R. Matzner, and R. Parker. Guidance for Use of the Index Reservoir in Drinking Water Exposure Assessments. Environmental Fate and Effects Division, Office of Pesticide Programs, U.S. Environmental Protection Agency, Washington, DC.

EPA. 2004. Abel, S.A. Pesticide Root Zone Model (PRZM) Field and Orchard Crop Scenarios: Guidance for Selecting Field Crop and Orchard Scenario Input Parameters. Environmental Fate and Effects Division, Office of Pesticide Programs, U.S. Environmental Protection Agency, Washington, DC. Online at: www.epa.gov/oppefed1/models/water/przm_scenario_metadata.wpd.

EPA. 2005. Irrigation Guidance for developing PRZM Scenarios. June 15, 2005.

Haan, C.T. and B.J. Barfield. 1978. Hydrology and Sedimentology of Surface Mined Lands. Office of Continuing Education and Extension, College of Engineering, University of Kentucky, Lexington KY 40506. pp 286.

NOAA. 2005. Comparative Climatic Data – Bakersfield, CA Mean Annual Precipitation: 1971-2000. National Oceanic and Atmospheric Administration (NOAA), National Weather Service Forecast Office. Online at: http://www.ncdc.noaa.gov/oa/climate/online/ccd/nrmpcp.txt; and http://www1.ncdc.noaa.gov/pub/data/ccd-data/CCD_2005.pdf

NOAA. 2006. National Climatic Data Center. Climate Data Inventories – Bakersfield, CA Station. Online at: http://www4.ncdc.noaa.gov/cgi-win/wwcgi.dll?wwDI~StnSrch~StnID~20002050.

TWCII. 2006. Averages and Records for Bakersfield, CA. The Weather Channel Interactive, Inc. Online at: http://www.weather.com and http://www.weather.com/outlook/recreation/outdoors/wxclimatology/monthly/93307.

University of California. Fruit and Nut Research Center. 1995. Olive Fact Sheet. Online at: http://fruitsandnuts.ucdavis.edu/crops/olive_factsheet.shtml

University of California. Fruit and Nut Research Center. 2006. Olives. Online at: http://fruitsandnuts.ucdavis.edu/crops/olive.shtml.

USDA.  1997.  Official Series Description – PORTERVILLE Series.  U.S. Department of Agriculture, Natural Resources Conservation Service (NRCS).  Dec. 2006.  Online at: http://ortho.ftw.nrcs.usda.gov/osd/dat/P/PORTERVILLE.html.

USDA. 1990. Davis, F.M., R.A. Leonard, W.G. Knisel. GLEAMS User Manual, Version 1.8.55. U.S. Department of Agriculture, Agricultural Research Service (ARS), Southeast Watershed Research Laboratory, Tifton, GA. SEWRL-030190FMD.

USDA. 1999. Crop Profile for Olives in California. U.S. Department of Agriculture, Pest Management Centers. Online at: http://www.ipmcenters.org/CropProfiles/docs/Caolives.html

USDA. 2000. Revised Universal Soil Loss Equation (RUSLE) EPA Pesticide Project. U.S. Department of Agriculture, National Resources Conservation Service (NRCS) and Agricultural Research Service (ARS).

USDA. 2002. 2002 Census of Agriculture. U.S. Department of Agriculture, National Agricultural Statistics Service (NASS). Online at: http://www.nass.usda.gov/census/census02/preliminary/2002censusdates.htm.

Contacts:

Bill Peacock

Farm Advisor

University of California Cooperative Extension – Tulare County

4437-B S. Laspina St.

Tulare, CA   93274

Phone: (559) 685-3309 Ext. 217

Fax: (559) 685-3319

Email: wlpeacock@ucdavis.edu