Iowa Corn

The Iowa corn scenario was developed to represent an environment in which field corn is grown in an area where rainfall is high and soils are more vulnerable to runoff than most places where corn is grown in the state. Based on the 2002 USDA National Agricultural Statistics Summary, Iowa has the most acreage of corn in the nation with 11,761,392 acres (USDA, 2005), representing 17% of the total corn acreage in the United States. Besides field corn, two other crops are in the species of Zea mays: popcorn and sweet corn. This scenario, though, is not intended to be used for these other two forms of corn as they have different cultural practices and tend to be more prominent in other parts of the country than in the Midwest.

Corn culture is approximately equally distributed across Iowa from east to west, but is most concentrated in the northern half of the state. Of 77 counties in Iowa, 24 are each planted with between 150,000 and 170,000 acres of corn, while just four have more than 170,000 acres.  Kossuth County, along the northern border of the state has the most acreage of corn, with 287,000 acres.

Table 1 lists the candidate weather stations for Iowa, along with the annual mean precipitation at each station. Based on data from weather.com, Table 1 also lists the mean precipitation in June, the highest runoff month in the Midwest. Because the corn culture is widespread across the state, a site near one of the weather stations (Moline, IL), was selected.  This site is on the Iowa-Illinois border and has the greatest annual rainfall of the candidate weather stations and the second highest rainfall in June.

| Table 1. Candidate Weather Stations for the Iowa Corn Scenario. |  |  |  |
| --- | --- | --- | --- |
| Station ID | Location | Annual Average Precipitation | June Average Precipitation |
| W14933 | Des Moines, IA | 33.4 in | 4.6 in |
| W14940 | Mason City, IA | 34.6 in | 5.1 in |
| W14943 | Sioux City, IA | 26.0 in | 3.6 in |
| W94810 | Waterloo, IA | 33.2 in | 4.8 in |
| W14923* | Moline, IL | 38.0 in | 4.6 in |

* The Moline, IL weather station was selected to represent this scenario

The site selected for this scenario is in Clinton County and is about 30 miles north of Moline. Clinton County has the sixth highest corn acreage in Iowa, with 169,070 acres or 1.4% of the acreage in the state. The site is in MLRA 105, the Northern Mississippi Valley Loess Hills. Soils in this area have dominantly formed in loess or loess of residuum (USDA, 2006).  Corn, soybeans, other grains for feed, and hay are the dominant crops. Clinton County, which is located in the Upper Mississippi River Basin, is drained by the Wapsipinicon and Maquoketa Rivers.

The soil selected for this scenario is the Fayette silty clay loam, a fine silty, mixed, superactive, mesic Typic Hapludalf. The Fayette soil is a benchmark soil. It is important to note that the Fayette soil is from Hydrologic Group B rather than Group C, which is usually selected, according to guidance (USEPA 2004). However, Groups C and D soils only represent 2.5% of the acreage in Clinton County, and are generally uncommon across the northern portion of the state, where most of the corn is grown in Iowa. Because these more runoff-prone soils are uncommon in this region, a Hydrologic Group B soil was chosen to represent the site. Fayette soils cover 22.9% of Clinton County, IA. Slopes range from 2 to 40%, although the greater slopes are severely eroded.  Slopes above 18% are not currently being used for corn agriculture to any great extent.

| Table 2. PRZM 3.12.2 Climate and Time Parameters for Clinton County Corn in Iowa. |  |  |
| --- | --- | --- |
| Parameter | Value | Source/Comments |
| Starting Date | Jan. 1, 1961 | Meteorological File from Moline, IL (W14923) |
| Ending Date | Dec. 31, 1990 | Meteorological File (W14923) |
| Pan Evaporation Factor (PFAC) | 0.76 | PRZM Manual Figure 5.1. (USEPA, 2006) |
| Snowmelt Factor (SFAC) | 0.36 cm °C-1 | Maximum value of minimum range of PRZM Manual Table 5.1 (USEPA, 2006) |
| Minimum Depth of Evaporation (ANETD) | 17.5 cm | PRZM Manual (USEPA, 2006); average of 15-20 cm |
| Pan Factor Flag (IPEIND) | 0 | Pan Factor Flag set to read from weather data as per guidance (USEPA, 2004) |

| Table 3. PRZM 3.12.2 Erosion and Landscape Parameters for Corn in Clinton County, Iowa. |  |  |
| --- | --- | --- |
| Parameter | Value | Source/Comments |
| Method to Calculate Erosion (ERFLAG) | 4 (MUSS) | PRZM Manual (USEPA, 2006) |
| USLE K Factor (USLEK) | 0.37 tons EI-1* | USDA NRCS Soil Data Mart (http://soildatamart.nrcs.usda.gov/) |
| USLE LS Factor (USLELS) | 3.73 | Value listed for 12.2% slope at 400 feet using equation in Hann and Barfield (1978) |
| USLE P Factor (USLEP) | 0.8 | 12.2% slope row crop PRZM Manual Table 5.6 (USEPA, 2006) |
| Field Area (AFIELD) | 172 ha | Area of Shipman Reservoir watershed (USEPA, 1999) |
| NRCS Hyetograph (IREG) | 3 | PRZM Manual Figure 5.8 (USEPA, 2006) Type 2, IREG=3 |
| Slope (SLP) | 6% | Set to 6%, as slope is greater than 6%, as per guidance (USEPA, 2004) |
| Hydraulic Length (HL) | 600 m | Shipman Reservoir (USEPA, 1999) |
| Irrigation Flag (IRFLAG) | 0 | Irrigation off |
| * EI = 100 ft-tons * in/ acre*hr |  |  |

| Table 4. PRZM 3.12.2 Crop Parameters for Corn in Clinton County, Iowa. |  |  |
| --- | --- | --- |
| Parameter | Value | Source/Comments |
| Initial Crop (INICRP) | 1 | Set to one for all crops (EPA, 2004). |
| Initial Surface Condition (ISCOND) | 1 | 1= fallow, default – parameter is ignored as ERFLAG > 0. |
| Number of Different Crops (NDC) | 1 | Set to number of crops in simulation. |
| Number of Cropping Periods (NCPDS) | 30 | Set to weather data in meteorological file: Moline, IL (W14923). |
| Maximum Rainfall Interception Storage of Crop (CINTCP) | 0.25 cm | Low end of corn heavy canopy; PRZM Table 5-4. |
| Maximum Active Root Depth (AMXDR) | 90 cm | Middle of range for corn, Table 5.9 |
| Maximum Canopy Coverage (COVMAX) | 100% | Set to 100 for row crops, as per guidance |
| Soil Surface Condition After Harvest (ICNAH) | 3 | 3 = residue, as according to guidance (USEPA, 2004) |
| Date of Crop Emergence (EMD, EMM, IYREM) | 25/05/61 | Middle of most active period; Associated with first RUSLE date of 10 days after planting. |
| Date of Crop Maturity (MAD, MAM, IYRMAT) | 24/07/61 | 60 days after planting (USDA, 1997) |
| Date of Crop Harvest (HAD, HAM, IYRHAR) | 19/10/61 | Middle of most active period (USDA, 1997) |
| Maximum Dry Weight (WFMAX) | 0.0 | Not used in scenario |
| Maximum Crop Height (HTMAX) | 300 cm | Upper end for corn from PRZM Manual Table 5.16 |
| SCS Curve Number (CN) | 86, 79, 86 | PRZM Table 5.10, B Soil, fallow, and row crop, contoured, poor condition |
| Manning’s N Value (MNGN) | 0.014 | RUSLE Project; M99CGCSN; corn for grain, Des Moines weather station; mulch tillage(USDA, 2000); dates adjusted to match planting and harvest |
| USLE C Factor (USLEC) | 0.016-0.307 | RUSLE Project; M99CGCSN; corn for grain, Des Moines weather station; mulch tillage(USDA, 2000); dates adjusted to match planting and harvest |

| Table 5. PRZM 3.12.2 Fayette Silty Clay Loam Soil Parameters for Corn in Clinton County, Iowa. |  |  |
| --- | --- | --- |
| Parameter | Value | Source/Comments |
| Total Soil Depth (CORED) | 152 cm | NRCS Soil Data Mart (SDM) (http://soildatamart.nrcs.usda.gov) |
| Number of Horizons (NHORIZ) | 4 | NRCS Soil Data Mart (SDM) |
| Horizon Thickness (THKNS) | 10 cm (HORIZN = 1) 5 cm (HORIZN = 2) 125 cm (HORIZN = 3) 12 cm (HORIZN = 4) | NRCS Soil Data Mart (SDM). The top horizon was split into two horizons as per PRZM Scenario Guidance (EPA, 2004). |
| Bulk Density (BD) | 1.4 g/cm3 (HORIZN = 1) 1.4 g/cm3 (HORIZN = 2) 1.37 g/cm3 (HORIZN = 3) 1.48 g/cm3 (HORIZN = 4) | NRCS Soil Data Mart (SDM) (http://soildatamart.nrcs.usda.gov). Midpoint of the reported range. PRZM Scenario Guidance (EPA, 2004). |
| Initial Water Content (THETO) | 0.310 cm3/cm3 (HORIZN =1) 0.310 cm3/cm3 (HORIZN =2) 0.279 cm3/cm3 (HORIZN = 3) 0.309 cm3/cm3 (HORIZN = 4) | NRCS Soil Data Mart (SDM); values are 15 bar water plus mean of available water capacity. |
| Compartment Thickness (DPN) | 0.1 cm (HORIZN = 1) 1 cm (HORIZN = 2) 5 cm (HORIZN = 3) 6 cm (HORIZN = 4) | NRCS Soil Data Mart (SDM) (http://soildatamart.nrcs.usda.gov). PRZM Scenario Guidance (EPA, 2004). |
| Field Capacity (THEFC) | 0.310 cm3/cm3 (HORIZN =1) 0.310 cm3/cm3 (HORIZN =2) 0.279 cm3/cm3 (HORIZN = 3) 0.309 cm3/cm3 (HORIZN = 4) | NRCS Soil Data Mart (SDM); values are 15 bar water plus mean of available water capacity |
| Wilting Point (THEWP) | 0.120 cm3/cm3 (HORIZN =1) 0.120 cm3/cm3 (HORIZN =2) 0.079 cm3/cm3 (HORIZN = 3) 0.119 cm3/cm3 (HORIZN = 4) | NRCS Soil Data Mart (SDM), soil characterization data; values are mean 15-bar water contents of Fayette silty clay loam |
| Organic Carbon Content (OC) | 0.93% (HORIZN = 1) 0.93% (HORIZN = 2) 0.14% (HORIZN = 3) 0.14% (HORIZN = 4) | NRCS SDM; values for horizons 1 to 3 = mean %OM / 1.724. PRZM Scenario Guidance (EPA, 2004). |

References

Haan, C.T., and B. J. Barfield.  1978.  Hydrology and Sedimentology of Surface Mined Lands.  Office of Continuing Education and Extension, College of Engineering, University, University of Kentucky, Lexington, KY 40506. pp 286.

USDA. 1997. Usual Planting and Harvest Dates for U. S. Field Crops. http://www.usda.gov/nass/pubs/uph97.htm

USDA. 2006.  Land Resource Regions and Major Land Resource Areas of the United States, the Caribbean, and the Pacific Basin. USDA Handbook 296. ftp://ftp-fc.sc.egov.usda.gov/NSSC/Ag_Handbook_296/Handbook_296_low.pdf

USDA.  2000.  Revised Universal Soil Loss Equation (RUSLE) EPA Pesticide Project.  U.S. Department of Agriculture, National Resources Conservation Service (NRCS) and Agricultural Research Service (ARS).

USDA.  2005. 2002 Census of Agriculture.  U.S. Department of Agriculture, National Agricultural Statistics Service (NASS).  Online at: http://www.nass.usda.gov/index.asp.

USDA.  2006.  Official Series Description – Fayette Series.  U.S. Department of Agriculture, Natural Resources Conservation Service (NRCS).  Sept. 2006.  Online at: http://ortho.ftw.nrcs.usda.gov/osd/dat/F/FAYETTE.html

USEPA.  1999.  Jones, R.D., J. Breithaupt, J. Carleton, L. Libelo, J. Lin, R. Matzner, and R. Parker.  Guidance for Use of the Index Reservoir in Drinking Water Exposure Assessments. Environmental Fate and Effects Division, Office of Pesticide Programs, U.S. Environmental Protection Agency, Washington, DC.

USEPA.  2004.  Abel, S.A.  Procedure for Conducting Quality Assurance and Quality Control of Existing and New PRZM Field and Orchard Crop Standard Scenarios.  Environmental Fate and Effects Division, Office of Pesticide Programs, U.S. Environmental Protection Agency, Washington, DC.

USEPA.  2006.  Carsel, R.F., J.C. Imhoff, P.R. Hummel, J.M. Cheplick, and A.S. Donigian, Jr.  PRZM-3, A Model for Predicting Pesticide and Nitrogen Fate in the Crop Root and Unsaturated Soil Zones: Users Manual for Release 3.12.2.  National Exposure Research Laboratory, Office of Research and Development, U.S. Environmental Protection Agency, Athens, GA.