Indiana Corn

The Indiana corn scenario was developed to represent an environment in which field corn is grown in an area where rainfall is high and soils are more vulnerable to runoff than most places in the state. Based on the 2002 USDA National Agricultural Statistics Summary, Indiana has the fifth most acreage of corn in the nation with 5,123,291 acres (USDA, 2005), representing 7.5% of the total acreage of corn in the United States.  Besides field corn, two other crops are in the species of Zea mays: popcorn and sweet corn. However, this scenario is not intended to be used for either of these two forms of corn as they have different cultural practices and tend to be more prominent in other parts of the country than the Midwest.

Rainfall tends to increase in Indiana from north to south with Evansville, along the southern border of the state, having the most rain.  Indianapolis in the center of the state receives the second most rain, while the two more northern weather stations receive less rain. Corn acreage, however, tends to be more concentrated in the northeast and north central parts of the state.  The four counties with the greatest acreage (Jasper, Clinton, Benton and Montgomery) are found in this quadrant of the state.

Table 1 lists the candidate weather stations for Indiana along with the annual mean precipitation for each station. Based on data from weather.com, Table 1 also lists the mean precipitation in June, the highest runoff month in most places in the Midwest.  The site selected for the Indiana corn scenario is in Clinton County.  This county is in the Wabash watershed of the Ohio River Basin. Weather from the Indianapolis weather station, which is representative of areas in Indiana where corn production is most intense, was used in this scenario.  While Evansville has the wettest weather, corn agriculture is not particularly intensive along the southern part of the state.  Clinton County has the second highest acreage of any county in Indiana with 118,033 acres, or 2.3% of the corn grown in the state.

| Table 1. Candidate Weather Stations in Indiana. |  |  |  |
| --- | --- | --- | --- |
| Station ID | Location | Annual Average Precipitation | June Average Precipitation |
| W14827 | Fort Wayne, IN | 36.6 in | 4.0 in |
| W14848 | South Bend, IN | 39.7 in | 4.2 in |
| W93817 | Evansville, IN | 45.8 in | 4.2 in* |
| W93819*** | Indianapolis, IN | 41.0 in | 4.1 in** |
| * mean monthly rainfall in May in Evansville is 4.8 in. ** mean monthly rainfall in July in Indianapolis is 4.4 in *** the selected weather station is Indianapolis, IN. |  |  |  |

The site for this scenario is located in MLRA 111D, the Indiana and Ohio Till Plain, Western Part of Clinton County.  This area is dominated by loess hills and flats, broken in places by glacial features such as kames, moraines, and outwash plains. The slopes in Clinton County range from 0 to 3%, and the soils have dominantly formed in loess, outwash, till or alluvium (USDA, 2006).  Corn, soybeans, other grains for feed, and hay are the dominant crops.

The soil selected for the scenario is the Fincastle silt loam, which is extensive in Clinton County and covers 25.8% of the land area. The Fincastle soil is a benchmark soil in Hydrologic Group C.

| Table 2. PRZM 3.12.2 Climate and Time Parameters for Clinton County, IN corn scenario. |  |  |
| --- | --- | --- |
| Parameter | Value | Source/Comments |
| Starting Date | Jan. 1, 1961 | Meteorological File from Indianapolis, IN (W 93819) |
| Ending Date | Dec. 31, 1990 | Meteorological File from Indianapolis, IN (W 93819) |
| Pan Evaporation Factor (PFAC) | 0.79 | PRZM Manual Figure 5.1 (USEPA, 2006) |
| Snowmelt Factor (SFAC) | 0.36 cm °C | Maximum value of minimum range of PRZM Manual Table 5.1 (USEPA, 2006) |
| Minimum Depth of Evaporation (ANETD) | 17.5 cm | PRZM Manual (USEPA, 2006); average of 15-20 cm |
| Pan Factor Flag (IPEIND) | 0 | Pan Factor Flag set to read from weather data as per guidance |

| Table 3. PRZM 3.12.2 Erosion and Landscape Parameters for Corn in Clinton County, Indiana |  |  |
| --- | --- | --- |
| Parameter | Value | Source/Comments |
| Method to Calculate Erosion (ERFLAG) | 4 (MUSS) | PRZM Manual (USEPA, 2006) |
| USLE K Factor (USLEK) | 0.49 tons EI-1* | USDA NRCS Soil Data Mart (http://soildatamart.nrcs.usda.gov/) |
| USLE LS Factor (USLELS) | 0.23 | Value listed for 1.4% slope at 400 feet using equation in Hann and Barfield (1978) |
| USLE P Factor (USLEP) | 0.6 | 1.4% slope row crop; PRZM Manual; Table 5.6 (USEPA, 2006), contouring |
| Field Area (AFIELD) | 172 ha | Area of Shipman Reservoir watershed (EPA, 1999) |
| NRCS Hyetograph (IREG) | 3 | PRZM Manual Figure 5.8 (USEPA, 2006) Type II, IREG=3 |
| Slope (SLP) | 1.4% | Mean value for Fincastle silty clay loam |
| Hydraulic Length (HL) | 600 m | Shipman Reservoir (EPA, 1999) |
| Irrigation Flag (IRFLAG) | 0 | Irrigation off |
| * EI = 100 ft-tons * in/ acre*hr |  |  |

| Table 4. PRZM 3.12.2 Crop Parameters for Corn in Clinton County, Indiana |  |  |
| --- | --- | --- |
| Parameter | Value | Source/Comments |
| Initial Crop (INICRP) | 1 | Set to 1 for all crops (EPA, 2004). |
| Initial Surface Condition (ISCOND) | 1 | 1= fallow, default – parameter is ignored as ERFLAG > 0. |
| Number of Different Crops (NDC) | 1 | Set to number of crops in simulation. |
| Number of Cropping Periods (NCPDS) | 30 | Set to weather data in meteorological file: Indianapolis, IN (W93819). |
| Maximum Rainfall Interception Storage of Crop (CINTCP) | 0.25 cm | Low end of corn heavy canopy; PRZM Table 5-4. |
| Maximum Active Root Depth (AMXDR) | 90 cm | Middle of range for corn, Table 5.9 |
| Maximum Canopy Coverage (COVMAX) | 100% | Set to 100 for row crops, as per guidance (USEPA, 2004) |
| Soil Surface Condition After Harvest (ICNAH) | 3 | 3 = residue, as according to guidance (USEPA, 2004) |
| Date of Crop Emergence (EMD, EMM, IYREM) | 15/05/61 | Consistent with RUSLE dates (USDA, 2000); Ma3CGSBM |
| Date of Crop Maturity (MAD, MAM, IYRMAT) | 14/07/61 | 60 days after emergence |
| Date of Crop Harvest (HAD, HAM, IYRHAR) | 20/10/61 | Consistent with RUSLE dates (USDA, 2000) Ma3CGSBM |
| Maximum Dry Weight (WFMAX) | 0.0 | Not used in scenario |
| Maximum Crop Height (HTMAX) | 300 cm | Upper end for corn from PRZM Manual Table 5.16 |
| SCS Curve Number (CN) | 91, 84, 91 | PRZM Table 5.10, C Soil, fallow, and row crop, contoured, poor condition |
| Manning’s N Value (MNGN) | 0.014 | RUSLE Project; Ma3CGSBM; corn for grain, Springfield, IL weather station; mulch tillage(USDA, 2000); dates adjusted to match planting and harvest |
| USLE C Factor (USLEC) | 0.017-0.510 | RUSLE Project; Ma3CGSBM; corn for grain, Springfield, IL weather station; mulch tillage(USDA, 2000); dates adjusted to match planting and harvest |

| Table 5. PRZM 3.12.2 Fincastle silt oam Soil Parameters for Corn in Clinton County, Indiana |  |  |
| --- | --- | --- |
| Parameter | Value | Source/Comments |
| Total Soil Depth (CORED) | 180 cm | NRCS Soil Data Mart (SDM) (http://soildatamart.nrcs.usda.gov) |
| Number of Horizons (NHORIZ) | 5 | NRCS Soil Data Mart (SDM) |
| Horizon Thickness (THKNS) | 10 cm (HORIZN = 1) 24 cm (HORIZN = 2) 48 cm (HORIZN = 3) 68 cm (HORIZN = 4) 30 cm (HORIZN = 5) | NRCS Soil Data Mart (SDM). The top horizon was split into two horizons as per PRZM Scenario Guidance (USEPA, 2004). |
| Bulk Density (BD) | 1.45 g/cm3 (HORIZN = 1) 1.45 g/cm3 (HORIZN = 2) 1.5 g/cm3 (HORIZN = 3) 1.7 g/cm3 (HORIZN = 4) 1.88 g/cm3 (HORIZN = 5) | NRCS Soil Data Mart (SDM) (http://soildatamart.nrcs.usda.gov). Midpoint of the reported range. PRZM Scenario Guidance (EPA, 2004). |
| Initial Water Content (THETO) | 0.292 cm3/cm3 (HORIZN =1) 0.292 cm3/cm3 (HORIZN =2) 0.235 cm3/cm3 (HORIZN = 3) 0.194 cm3/cm3 (HORIZN = 4) 0.169 cm3/cm3 (HORIZN = 5) | NRCS Soil Data Mart (SDM); values are mean 1/3-bar water contents of Fincastle silty clay loam soils. |
| Compartment Thickness (DPN) | 0.1 cm (HORIZN = 1) 2 cm (HORIZN = 2) 3 cm (HORIZN = 3) 4 cm (HORIZN = 4) 5 cm (HORIZN = 5) | NRCS Soil Data Mart (SDM) (http://soildatamart.nrcs.usda.gov). PRZM Scenario Guidance (EPA, 2004). |
| Field Capacity (THEFC) | 0.292 cm3/cm3 (HORIZN =1) 0.292 cm3/cm3 (HORIZN =2) 0.235 cm3/cm3 (HORIZN = 3) 0.194 cm3/cm3 (HORIZN = 4) 0.169 cm3/cm3 (HORIZN = 5) | NRCS Soil Data Mart (SDM); values are mean available water plus the wilting point water content of Fincastle silty clay loam soils. |
| Wilting Point (THEWP) | 0.082 cm3/cm3 (HORIZN =1) 0.082 cm3/cm3 (HORIZN =2) 0.065 cm3/cm3 (HORIZN = 3) 0.114 cm3/cm3 (HORIZN = 4) 0.139 cm3/cm3 (HORIZN = 5) | NRCS Soil Data Mart (SDM) Soil Characterization data; values are mean 15-bar water contents of Fincastle silty clay loam soils. |
| Organic Carbon Content (OC) | 1.16% (HORIZN = 1) 1.16% (HORIZN = 2) 0.44% (HORIZN = 3) 0.44% (HORIZN = 4) 0.44% (HORIZN = 5) | NRCS SDM; values for horizons 1 to 3 = mean %OM / 1.724. PRZM Scenario Guidance (USEPA, 2004). |

References

Haan, C.T., and B. J. Barfield.  1978.  Hydrology and Sedimentology of Surface Mined Lands.  Office of Continuing Education and Extension, College of Engineering, University, University of Kentucky, Lexington, KY 40506. pp 286.

USDA.  2000.  Revised Universal Soil Loss Equation (RUSLE) EPA Pesticide Project.  U.S. Department of Agriculture, National Resources Conservation Service (NRCS) and Agricultural Research Service (ARS).

USDA.  2005. 2002 Census of Agriculture.  U.S. Department of Agriculture, National Agricultural Statistics Service (NASS).  Online at: http://www.nass.usda.gov/index.asp.

USDA.  2003.  Official Series Description – Fincastle Series.  U.S. Department of Agriculture, Natural Resources Conservation Service (NRCS).  Dec. 2006.  Online at: http://ortho.ftw.nrcs.usda.gov/osd/dat/F/FINCASTLE.html

USEPA. 1999. Jones, R.D., J. Breithaupt, J. Carleton, L. Libelo, J. Lin, R. Matzner, and R. Parker. Guidance for Use of the Index Reservoir in Drinking Water Exposure Assessments. Environmental Fate and Effects Division, Office of Pesticide Programs, U.S. Environmental Protection Agency, Washington, DC.

USEPA.  2004.  Abel, S.A.  Procedure for Conducting Quality Assurance and Quality Control of Existing and New PRZM Field and Orchard Crop Standard Scenarios.  Environmental Fate and Effects Division, Office of Pesticide Programs, U.S. Environmental Protection Agency, Washington, DC.

USEPA.  2006.  Carsel, R.F., J.C. Imhoff, P.R. Hummel, J.M. Cheplick, and A.S. Donigian, Jr.  PRZM-3, A Model for Predicting Pesticide and Nitrogen Fate in the Crop Root and Unsaturated Soil Zones: Users Manual for Release 3.12.2.  National Exposure Research Laboratory, Office of Research and Development, U.S. Environmental Protection Agency, Athens, GA.