Kansas Corn

The Kansas corn scenario was developed to represent an environment in which field corn is grown in an area where rainfall is high and soils are more vulnerable to runoff than most places where corn is grown in the state. Based on the 2002 USDA National Agricultural Statistics Summary, Kansas has the tenth most acreage of corn in the nation, with 2,494,179 acres (USDA, 2005), representing 3.6% of the total corn acreage in the United States. Besides field corn, two other crops are in the same species of Zea mays: popcorn and sweet corn. This scenario is not intended to be used for these other two forms of corn as they have different cultural practices and tend to be more prominent in other parts of the country than in the Midwest.

Kansas has a strong east to west rainfall gradient. The western part of the state is semi-arid (Dodge City and Goodland), while the eastern part of the state (Topeka and Wichita) is humid. Corn culture in Kansas is localized in two regions: the northeast corner and the southwestern part of the state over the Edwards Aquifer, south of Dodge City and Garden City. In general, occurrence of pesticide runoff from use on irrigated corn fields is less than in areas which rely on natural rainfall.  Although more corn is grown in the southwestern portion of the state, it is almost all irrigated and there is little surface water. In fact, several creeks and rivers that enter the state from Colorado are dry in most years (e.g., White Woman Creek, Bear Creek). Thus, surface water is not expected to be affected by chemical runoff in the southwestern part of the state. For these reasons, a site in the eastern corn-growing region of Kansas was selected for assessing vulnerable drinking water and aquatic wildlife in the state.

Table 1 lists the candidate weather stations for Kansas along with the annual mean precipitation at each station.  Based on data from weather.com, Table 1 also lists the mean precipitation in June, the highest runoff month in most parts of in the Midwest. The eastern corn growing region in the state is closer to Topeka than Wichita, and thus the Topeka weather station was selected to represent the site. The selected site for this scenario is in Brown County, which is the county with the second most corn acreage.  The corn acreage in this site is the most un-irrigated (96, 118 acres) and represents 3.8% of the corn grown in the state.

| Table 1. Candidate Weather Stations in Kansas. |  |  |  |
| --- | --- | --- | --- |
| Station ID | Location | Annual Average Precipitation | June Average Precipitation |
| W03928 | Wichita, KS | 30.4 in | 4.2 in |
| W13985 | Dodge City, KS | 22.4 in | 3.2 in |
| W13996* | Topeka, KS | 35.6 in | 4.9 in |
| W23065 | Goodland, KS | 19.8 in | 3.3 in |

*  Topeka weather station was selected to represent this scenario.

The site is in MLRA M106, the Kansas and Nebraska Loess Drift Hills. This area is almost entirely dissected till plains. Soils in this area have dominantly formed in till, alluvium, or colluvium (USDA, 2006). Corn, soybeans, sorghum, and wheat are the dominant crops, but 22% of the land is grasslands.  Brown County is mostly in the Delaware River watershed of the Missouri Basin.

The soil selected for this scenario is the Wymore silty clay loam.  The Wymore soil is not a benchmark soil, but is common in Brown County and covers 34% of the land area. It is important to note that the Wymore soil is from Hydrologic Group D rather than C, which is usually selected according to EPA guidance (USEPA, 2004). However, since the Wymore soil itself covers over one-third of the county, a D soil was thought to be more appropriate for representing the runoff potential of vulnerable soils in this area. Slopes at this site range from 0 to 6%, although the steeper slopes are eroded.

| Table 2. PRZM 3.12.2 Climate and Time Parameters for Corn in Brown County, KS . |  |  |
| --- | --- | --- |
| Parameter | Value | Source/Comments |
| Starting Date | Jan. 1, 1961 | Meteorological File from Topeka, KS (W13996) |
| Ending Date | Dec. 31, 1990 | Meteorological File from Topeka, KS (W13996) |
| Pan Evaporation Factor (PFAC) | 0.72 | PRZM Manual Figure 5.1. (USEPA, 2006) |
| Snowmelt Factor (SFAC) | 0.36 cm °C-1 | Maximum value of minimum range of PRZM Manual Table 5.1 (USEPA, 2006) |
| Minimum Depth of Evaporation (ANETD) | 17.5 cm | PRZM Manual (USEPA, 2006) Average of 15-20 cm |
| Pan Factor Flag (IPEIND) | 0 | Pan Factor Flag set to read from weather data as per guidance (USEPA, 2004) |

| Table 3. PRZM 3.12.2 Erosion and Landscape Parameters for Brown County, KS Corn Scenario. |  |  |
| --- | --- | --- |
| Parameter | Value | Source/Comments |
| Method to Calculate Erosion (ERFLAG) | 4 (MUSS) | PRZM Manual (USEPA, 2006) |
| USLE K Factor (USLEK) | 0.37 tons EI-1* | USDA NRCS Soil Data Mart (http://soildatamart.nrcs.usda.gov/) |
| USLE LS Factor (USLELS) | 0.58 | Value listed for 3.4% slope at 400 feet using equation in Hann and Barfield (1978) |
| USLE P Factor (USLEP) | 0.5 | 3.4 % slope row crop with contouring PRZM Manual Table 5.6 (USEPA, 2006) |
| Field Area (AFIELD) | 172 ha | Area of Shipman Reservoir watershed (USEPA, 1999) |
| NRCS Hyetograph (IREG) | 3 | PRZM Manual Figure 5.8 (USEPA, 2006) Type I, IREG=3 |
| Slope (SLP) | 3.4% | acreage-weighted mean value for Wymore silty clay loam |
| Hydraulic Length (HL) | 600 m | Shipman Reservoir (USEPA, 1999) |
| Irrigation Flag (IRFLAG) | 0 | No irrigation |
| * EI = 100 ft-tons * in/ acre*hr |  |  |

| Table 4. PRZM 3.12.2 Crop Parameters for Brown County, KS Corn Scenario. |  |  |
| --- | --- | --- |
| Parameter | Value | Source/Comments |
| Initial Crop (INICRP) | 1 | Set to 1 for all crops (EPA, 2004). |
| Initial Surface Condition (ISCOND) | 1 | 1= fallow; default parameter is ignored as ERFLAG > 0. |
| Number of Different Crops (NDC) | 1 | Set to number of crops in simulation. |
| Number of Cropping Periods (NCPDS) | 30 | Set to weather data in meteorological file: Topeka, KS (W13996). |
| Maximum Rainfall Interception Storage of Crop (CINTCP) | 0.25 cm | Low end of corn heavy canopy; PRZM Table 5-4. |
| Maximum Active Root Depth (AMXDR) | 90 cm | Middle of range for corn, Table 5.9 |
| Maximum Canopy Coverage (COVMAX) | 100% | Set to 100 for row crops, as per guidance |
| Soil Surface Condition After Harvest (ICNAH) | 3 | 3 = residue, as according to guidance |
| Date of Crop Emergence (EMD, EMM, IYREM) | 10/05/61 | Consistent with RUSLE dates (USDA, 2000); H88CGWWM |
| Date of Crop Maturity (MAD, MAM, IYRMAT) | 09/07/61 | 60 days after emergence |
| Date of Crop Harvest (HAD, HAM, IYRHAR) | 20/10/61 | Consistent with RUSLE dates (USDA, 2000) H88CGWWM |
| Maximum Dry Weight (WFMAX) | 0.0 | Not used in scenario |
| Maximum Crop Height (HTMAX) | 300 cm | Upper end for corn from PRZM Manual; Table 5.16 |
| SCS Curve Number (CN) | 94, 88, 94 | PRZM Table 5.10, D Soil, fallow, and row crop, contoured, poor condition |
| Manning’s N Value (MNGN) | 0.023 | RUSLE Project; H88CGWWM; corn for grain, Salina, KS weather station; mulch tillage(USDA, 2000); dates adjusted to match planting and harvest |
| USLE C Factor (USLEC) | 0.038-0.195 | RUSLE Project; H88CGWWM; corn for grain, Salina, KS weather station; mulch tillage(USDA, 2000); dates adjusted to match planting and harvest |

| Table 5. PRZM 3.12.2 Wymore Silty Clay Loam Soil Parameters for Corn in Brown County, KS |  |  |
| --- | --- | --- |
| Parameter | Value | Source/Comments |
| Total Soil Depth (CORED) | 92 cm | NRCS Soil Data Mart (SDM) (http://soildatamart.nrcs.usda.gov) |
| Number of Horizons (NHORIZ) | 4 | NRCS Soil Data Mart (SDM) |
| Horizon Thickness (THKNS) | 10 cm (HORIZN = 1) 8 cm (HORIZN = 2) 8 cm (HORIZN = 3) 66 cm (HORIZN = 4) | NRCS Soil Data Mart (SDM). The top horizon was split into two horizons as per PRZM Scenario Guidance (EPA, 2004). |
| Bulk Density (BD) | 1.30 g/cm3 (HORIZN = 1) 1.30 g/cm3 (HORIZN = 2) 1.30 g/cm3 (HORIZN = 3) 1.29 g/cm3 (HORIZN = 4) | NRCS Soil Data Mart (SDM) (http://soildatamart.nrcs.usda.gov). Midpoint of the reported range. PRZM Scenario Guidance (EPA, 2004). |
| Initial Water Content (THETO) | 0.364 cm3/cm3 (HORIZN =1) 0.364 cm3/cm3 (HORIZN =2) 0.274 cm3/cm3 (HORIZN = 3) 0.320 cm3/cm3 (HORIZN = 4) | NRCS Soil Data Mart (SDM); values are mean available water plus the wilting point water content of Wymore silty clay loam soils. |
| Compartment Thickness (DPN) | 0.1 cm (HORIZN = 1) 2 cm (HORIZN = 2) 2 cm (HORIZN = 3) 2 cm (HORIZN = 4) | NRCS Soil Data Mart (SDM) (http://soildatamart.nrcs.usda.gov). PRZM Scenario Guidance (EPA, 2004). |
| Field Capacity (THEFC) | 0.364 cm3/cm3 (HORIZN =1) 0.364 cm3/cm3 (HORIZN =2) 0.274 cm3/cm3 (HORIZN = 3) 0.320 cm3/cm3 (HORIZN = 4) | NRCS Soil Data Mart (SDM); values are mean available water plus the wilting point water content of Wymore silty clay loam soils. |
| Wilting Point (THEWP) | 0.139 cm3/cm3 (HORIZN =1) 0.139 cm3/cm3 (HORIZN =2) 0.224 cm3/cm3 (HORIZN = 3) 0.229 cm3/cm3 (HORIZN = 4) | NRCS Soil Data Mart (SDM) Soil Characterization data; values are mean 15-bar water contents of Wymore silty clay loam soils. |
| Organic Carbon Content (OC) | 1.74% (HORIZN = 1) 1.74% (HORIZN = 2) 1.30% (HORIZN = 3) 0.87% (HORIZN = 4) | NRCS SDM; values for horizons 1 to 3 = mean %OM / 1.724. PRZM Scenario Guidance (EPA, 2004). |

References

Haan, C.T., and B.J. Barfield.  1978.  Hydrology and Sedimentology of Surface Mined Lands.  Office of Continuing Education and Extension, College of Engineering, University of Kentucky, Lexington KY 40506.  pp 286.

USDA.  2000.  Revised Universal Soil Loss Equation (RUSLE) EPA Pesticide Project.  U.S. Department of Agriculture, National Resources Conservation Service (NRCS) and Agricultural Research Service (ARS).

USDA.  2005. 2002 Census of Agriculture.  U.S. Department of Agriculture, National Agricultural Statistics Service (NASS).  Online at: http://www.nass.usda.gov/index.asp.

USDA.  2003.  Official Series Description – Wymore Series.  U.S. Department of Agriculture, Natural Resources Conservation Service (NRCS).  Dec. 2006.  Online at: http://ortho.ftw.nrcs.usda.gov/osd/dat/W/WYMORE.html

USDA.  2006.  Soil Survey Areas of Fresno, Kings, Kern, Madera, and Merced Counties  U.S. Department of Agriculture, Natural Resources Conservation Service (NRCS), Soil Data Mart.  March 1, 2006.  Online at: http://soildatamart.nrcs.usda.gov.

USEPA.  1999.  Jones, R.D., J. Breithaupt, J. Carleton, L. Libelo, J. Lin, R. Matzner, and R. Parker.  Guidance for Use of the Index Reservoir in Drinking Water Exposure Assessments. Environmental Fate and Effects Division, Office of Pesticide Programs, U.S. Environmental Protection Agency, Washington, DC.

USEPA.  2004.  Abel, S.A.  Procedure for Conducting Quality Assurance and Quality Control of Existing and New PRZM Field and Orchard Crop Standard Scenarios.  Environmental Fate and Effects Division, Office of Pesticide Programs, U.S.

USEPA. 2006. Carsel, R.F., J.C. Imhoff, P.R. Hummel, J.M. Cheplick, and A.S. Donigian, Jr.  PRZM-3, A Model for Predicting Pesticide and Nitrogen Fate in the Crop Root and Unsaturated Soil Zones: Users Manual for Release 3.12.2.  National Exposure Research Laboratory, Office of Research and Development, U.S. Environmental Protection Agency, Athens, GA.