Michigan Melons

The Michigan melons scenario was developed to represent an environment in which melons are grown in an area where rainfall is high and soils are more vulnerable to runoff than most places where melons are grown in the state. The Michigan melon crops include cantaloupe, honeydew, and watermelon. Based on the 2002 USDA National Agricultural Statistics Summary, cantaloupe production accounted for approximately 70% of the crop acreage for the above three melon crops (USDA, 2005).  In 2002, 294,045 acres of melons were grown nationally.  Michigan ranked 23rd  in the nation in production of melons with 1,381 acres or 0.5% of the national acreage.

Melons belong to the cucurbit family of plants, known as Cucurbitaceae, which includes cucumbers, gourds, squash, and pumpkins. There are several different genus names used in the family. Cantaloupes (Cucumis melo L. var. antalupensis) and honeydews (Cucumis melo L. var. inodorus) are classified in the same genus. Mixed melons, including crenshaw, casaba, Santa Claus, Persian, Juan Canary, piel de sapo, and other melon types are in this genus, while watermelons are in a different genus Citrullus lanatus (Thunb.) Matsum & Nakai.

Melons in Michigan tend to be grown mostly in counties that are adjacent to the Great Lakes in the southern portion of the state. Monroe County has the largest melon production (236 acres) and is located in the southeast corner of the state along Lake Erie. Unlike the other states in the Midwest for which June is the wettest month, the wettest months in Michigan are August and September.

| Table 1. Candidate Weather Stations in Michigan. |  |  |  |
| --- | --- | --- | --- |
| Station ID | Location | Annual Average Precipitation | June Average Precipitation |
| W14826 | Flint, MI | 31.6 in | 3.1 in |
| W14836 | Lansing, MI | 31.5 in | 3.6 in |
| W14840 | Muskegon, MI | 32.9 in | 2.6 in |
| W14847 | Sault Ste. Marie, MI | 34.7 in | 3.0 in |
| W14850 | Traverse City, MI | 33.5 in | 3.3 in |
| W94814 | Houghton, MI | 33.8 in | 2.8 in |
| W94847* | Detroit, MI | 32.9 in | 3.6 in |
| W94849 | Alpena, MI | 28.4 in | 2.6 in |
| W94860 | Grand Rapids, MI | 37.1 | 3.7 in |

*  The weather station selected for the Michigan melon scenario is Detroit.

The candidate Michigan weather stations in the southern part of the state include Muskegon, Lansing, Detroit and Grand Rapids (Table 1). Since melons are mostly grown in counties adjacent to the Great Lakes, it is reasonable to assume that proximity to the lakes is an important consideration. Of these four cities, Muskegon is located directly on Lake Michigan, and Detroit is located on Lake St. Clair, about 10 miles north of Lake Erie.  Both locations have nearly identical precipitation with a mean annual precipitation of 32.9 inches. However, Detroit receives somewhat more rainfall in the spring while precipitation in Muskegon is greater in the fall. Since pesticide applications are more prevalent around the time of planting rather than harvest, melon fields near Detroit would be expected to generate more pesticide runoff than those near Muskegon.  For these reasons, Monroe County, located just south of Detroit, was chosen as the site for the Michigan melon scenario.

The site for this scenario is in MLRA L99, the Erie-Huron Lake Plain. This area is a nearly level lake plain with a few scattered sandy ridges, representing past shorelines and moraines.  Soils in this area have dominantly formed in till, moraines, and lake plains. (USDA, 2006).  Corn, soybeans, winter wheat and hay are the dominant crops in this area. Monroe County is part of the Great Lakes Basin.

The soil selected for the scenario is the Selfridge loamy sand, which is classified as loamy, mixed, active, mesic Aquic Arenic Hapludalf.  The Selfridge soil is a benchmark soil in Hydrologic Group B. Although Group C soils are extensive in Monroe County, melons are not tolerant of poorly drained soils, so a spatially extensive Group B soil was chosen to represent a vulnerable soil that is used for melon agriculture in this county. The Selfridge soil series description lists vegetables among the crops grown on the series. The Selfridge soil covers 12.9% of the land area in the county. Slopes range from 0 to 3% in this part of the county.

| Table 2. PRZM 3.12.2 Climate and Time Parameters for Monroe County, MI Melon Scenario. |  |  |
| --- | --- | --- |
| Parameter | Value | Source/Comments |
| Starting Date | Jan. 1, 1961 | Meteorological File from Detroit, MI (W94847) |
| Ending Date | Dec. 31, 1990 | Meteorological File from Detroit, MI (W94847) |
| Pan Evaporation Factor (PFAC) | 0.77 | PRZM Manual Figure 5.1. (USEPA, 2006) |
| Snowmelt Factor (SFAC) | 0.36 cm °C-1 | Maximum value of minimum range of PRZM Manual Table 5.1 (USEPA, 2006) |
| Minimum Depth of Evaporation (ANETD) | 17.5 cm | PRZM Manual (USEPA 2006) Average of 15-20 cm |
| Pan Factor Flag (IPEIND) | 0 | Pan Factor Flag set to read from weather data as per guidance (USEPA, 2004) |

USLE C factor and Manning’s N values are from a RUSLE assessment for mulch tilled strawberries (RUSLE Project; LA2STSTN) using weather data from Grand Rapids. This set  of values was chosen as it was in the same region of the country and the data set generated using strawberries was the most similar crop available for this area of the country.

| Table 3. PRZM 3.12.2 Erosion and Landscape Parameters for Monroe County, MI Melon Scenario. |  |  |
| --- | --- | --- |
| Parameter | Value | Source/Comments |
| Method to Calculate Erosion (ERFLAG) | 4 (MUSS) | PRZM Manual (USEPA, 2006) |
| USLE K Factor (USLEK) | 0.17 tons EI-1* | USDA NRCS Soil Data Mart (http://soildatamart.nrcs.usda.gov/) |
| USLE LS Factor (USLELS) | 0.25 | Value listed for 1.5% slope at 400 feet using equation in Hann and Barfield (1978) |
| USLE P Factor (USLEP) | 1 | No contouring(USEPA, 2006) |
| Field Area (AFIELD) | 172 ha | Area of Shipman Reservoir watershed (EPA, 1999) |
| NRCS Hyetograph (IREG) | 3 | PRZM Manual Figure 5.8 (USEPA, 2006) Type 2, IREG=3 |
| Slope (SLP) | 1.5% | Mean value for Selfridge loamy sand, weighted for acreage |
| Hydraulic Length (HL) | 600 m | Shipman Reservoir (EPA, 1999) |
| Irrigation Flag (IRFLAG) | 0 | Irrigation off |
| * EI = 100 ft-tons * in/ acre*hr |  |  |

| Table 4. PRZM 3.12.2 Crop Parameters for Monroe County, MI Melon Scenario. |  |  |
| --- | --- | --- |
| Parameter | Value | Source/Comments |
| Initial Crop (INICRP) | 1 | Set to 1 for all crops (EPA, 2004). |
| Initial Surface Condition (ISCOND) | 1 | 1= fallow; default parameter is ignored as ERFLAG > 0. |
| Number of Different Crops (NDC) | 1 | Set to number of crops in simulation. |
| Number of Cropping Periods (NCPDS) | 30 | Set to weather data in meteorological file: Detroit, MI (W94847). |
| Maximum Rainfall Interception Storage of Crop (CINTCP) | 0.25 cm | Moderate to heavy canopy PRZM table 5-4. Value consistent with TX melons (EPA, 2004). |
| Maximum Active Root Depth (AMXDR) | 46 cm | Kemble JK. Basics of crop irrigation. Alabama cooperative extension system. ANR-1169 April 2000 |
| Maximum Canopy Coverage (COVMAX) | 100% | Value consistent with TX melons (EPA, 2004). |
| Soil Surface Condition After Harvest (ICNAH) | 3 | 3 = residue. Melons are picked and packed directly from the vines in the field (USDA, 1998). |
| Date of Crop Emergence (EMD, EMM, IYREM) | 30/04/61 | USDA Crop Profile for cantaloupes in IN (1998). Plant seedlings, so emergence = planting. |
| Date of Crop Maturity (MAD, MAM, IYRMAT) | 25/06/61 | USDA Crop Profile for cantaloupes in IN (1998) |
| Date of Crop Harvest (HAD, HAM, IYRHAR) | 15/08/61 | USDA Crop Profile for cantaloupes in IN (1998) |
| Maximum Dry Weight (WFMAX) | 0.0 | Not used in scenario |
| Maximum Crop Height (HTMAX) | 25 cm | Value consistent with TX melons (EPA, 2004). |
| SCS Curve Number (CN) | 86, 79, 86 | PRZM Table 5.10, B Soil, fallow, and row crop, contoured, poor condition |
| Manning’s N Value (MNGN) | 0.040 | RUSLE Project; LA2STSTN; strawberries, Grand Rapids weather station; mulch tillage(USDA, 2000)., dates adjusted to match planting and harvest |
| USLE C Factor (USLEC) | 0.011-0.038 | RUSLE Project; LA2STSTN; strawberries, Grand Rapids weather station; mulch tillage(USDA, 2000). dates adjusted to match planting and harvest |

| Table 5. PRZM 3.12.2 Selfridge loamy sand Soil Parameters for Monroe County, MI Melon Scenario. |  |  |
| --- | --- | --- |
| Parameter | Value | Source/Comments |
| Total Soil Depth (CORED) | 151 cm | NRCS Soil Data Mart (SDM) (http://soildatamart.nrcs.usda.gov) |
| Number of Horizons (NHORIZ) | 5 | NRCS Soil Data Mart (SDM) |
| Horizon Thickness (THKNS) | 10 cm (HORIZN = 1) 10 cm (HORIZN = 2) 44 cm (HORIZN = 3) 9 cm (HORIZN = 4) 78 cm (HORIZN = 5) | NRCS Soil Data Mart (SDM). The top horizon was split into two horizons as per. PRZM Scenario Guidance (EPA, 2004). |
| Bulk Density (BD) | 1.32 g/cm3 (HORIZN = 1) 1.32 g/cm3 (HORIZN = 2) 1.45 g/cm3 (HORIZN = 3) 1.40 g/cm3 (HORIZN = 4) 1.60 g/cm3 (HORIZN = 5) | NRCS Soil Data Mart (SDM) (http://soildatamart.nrcs.usda.gov). Midpoint of the reported range. PRZM Scenario Guidance (USEPA, 2004). |
| Initial Water Content (THETO) | 0.159 cm3/cm3 (HORIZN =1) 0.159 cm3/cm3 (HORIZN =2) 0.139 cm3/cm3 (HORIZN = 3) 0.179 cm3/cm3 (HORIZN = 4) 0.148 cm3/cm3 (HORIZN = 5) | NRCS Soil Data Mart (SDM); values are mean available water plus the wilting point water content of Selfridge loamy sand soils |
| Compartment Thickness (DPN) | 0.1 cm (HORIZN = 1) 2 cm (HORIZN = 2) 2 cm (HORIZN = 3) 3 cm (HORIZN = 4) 3 cm (HORIZN = 5) | NRCS Soil Data Mart (SDM) (http://soildatamart.nrcs.usda.gov). PRZM Scenario Guidance (USEPA, 2004). |
| Field Capacity (THEFC) | 0.159 cm3/cm3 (HORIZN =1) 0.159 cm3/cm3 (HORIZN =2) 0.139 cm3/cm3 (HORIZN = 3) 0.179 cm3/cm3 (HORIZN = 4) 0.148 cm3/cm3 (HORIZN = 5) | NRCS Soil Data Mart (SDM); values are mean available water plus the wilting point water content of Selfridge loamy sand soils. |
| Wilting Point (THEWP) | 0.049 cm3/cm3 (HORIZN =1) 0.049 cm3/cm3 (HORIZN =2) 0.049 cm3/cm3 (HORIZN = 3) 0.042 cm3/cm3 (HORIZN = 4) 0.018 cm3/cm3 (HORIZN = 5) | NRCS Soil Data Mart (SDM) Soil Characterization data; values are mean 15-bar water contents of Selfridge loamy sand soils. |
| Organic Carbon Content (OC) | 1.16% (HORIZN = 1) 1.16% (HORIZN = 2) 0.145% (HORIZN = 3) 0.145% (HORIZN = 4) 0.145% (HORIZN = 5) | NRCS SDM; values for horizons 1 to 3 = mean %OM / 1.724. PRZM Scenario Guidance (USEPA, 2004). |

References

Haan, C.T., and B.J. Barfield.  1978.  Hydrology and Sedimentology of Surface Mined Lands.  Office of Continuing Education and Extension, College of Engineering, University of Kentucky, Lexington KY 40506.  pp 286.

USDA.  1998.  Crop Profile for Cantaloupes in Indiana.  U.S. Department of Agriculture, Pest Management Centers.  October 1999.  Online at: http://www.ipmcenters.org/CropProfiles/docs/Incantaloupes.pdf

USDA.  2000.  Revised Universal Soil Loss Equation (RUSLE) EPA Pesticide Project.  U.S. Department of Agriculture, National Resources Conservation Service (NRCS) and Agricultural Research Service (ARS).

USDA.  2005. 2002 Census of Agriculture.  U.S. Department of Agriculture, National Agricultural Statistics Service (NASS).  Online at: http://www.nass.usda.gov/index.asp.

USDA.  2003.  Official Series Description – Selfridge Series.  U.S. Department of Agriculture, Natural Resources Conservation Service (NRCS).  Dec. 2006.  Online at: http://ortho.ftw.nrcs.usda.gov/osd/dat/S/SELFRIDGE.html

USEPA.  1999.  Jones, R.D., J. Breithaupt, J. Carleton, L. Libelo, J. Lin, R. Matzner, and R. Parker.  Guidance for Use of the Index Reservoir in Drinking Water Exposure Assessments. Environmental Fate and Effects Division, Office of Pesticide Programs, U.S. Environmental Protection Agency, Washington, DC.

USEPA.  2004.  Abel, S.A.  Procedure for Conducting Quality Assurance and Quality Control of Existing and New PRZM Field and Orchard Crop Standard Scenarios.  Environmental Fate and Effects Division, Office of Pesticide Programs, U.S. Environmental Protection Agency, Washington, DC.

USEPA.  2006.  Carsel, R.F., J.C. Imhoff, P.R. Hummel, J.M. Cheplick, and A.S. Donigian, Jr.  PRZM-3, A Model for Predicting Pesticide and Nitrogen Fate in the Crop Root and Unsaturated Soil Zones: Users Manual for Release 3.12.2.  National Exposure Research Laboratory, Office of Research and Development, U.S. Environmental Protection Agency, Athens, GA.

http://www2.ftw.nrcs.usda.gov/osd/dat/S/SELFRIDGE.html