Nebraska Corn

The Nebraska corn scenario was developed to represent an environment in which field corn is grown in an area where rainfall is high and soils are more vulnerable to runoff than most places in the state. Based on the 2002 USDA National Agricultural Statistics Summary, Nebraska has the third highest acreage of corn in the nation, with 7,344,715 acres (USDA, 2005), representing 10.8% of the corn acreage in the United States. Besides field corn, two other crops are in the species of Zea mays: popcorn and sweet corn. However, this scenario is not intended to be used for either of these two other forms of corn as they have different cultural practices and tend to be more prominent in other parts of the country than the Midwest.

While irrigation extends corn culture west across Nebraska along the Platte River and its tributaries, unirrigated corn is predominantly grown along the eastern edge of the state. In general, occurrence of pesticides in surface waters from use on irrigated corn fields is less than in areas which rely on natural rainfall.  The two top counties in unirrigated acreage are just west of Omaha; Saunders and Lancaster. Since Omaha also has the most annual rainfall of the candidate weather stations (Table 1) and close to the highest average June precipitation, it was selected as the weather station for the scenario. The top unirrigated corn acreage county, Saunders, was selected as the location for the Nebraska corn scenario as it is due west of Omaha about 20 mi. There are 127,150 acres of unirrigated corn in Saunders County and a total of 180, 288 acres of corn (USDA, 2005). Saunders ranks third in corn acreage overall, behind York and Hamilton Counties.

| Table 1. Candidate Weather Stations in Nebraska for the NE Corn Scenario. |  |  |  |
| --- | --- | --- | --- |
| Station ID | Location | Annual Average Precipitation | June Average Precipitation |
| W14935 | Grand Island, NE | 25.9 in | 3.7 in |
| W14941 | Norfolk, NE | 26.7 in | 4.3 in |
| W24023 | North Platte, NE | 19.9 in | 3.2 in |
| W24028 | Scotts Bluff, NE | 16.3 in | 2.6 in |
| W94918 | Omaha, NE | 30.2 in | 4.0 in |

The site for this scenario is in MLRA M106, the Kansas and Nebraska Loess Drift Hills.  This area, which is located in the Platte River watershed of the Missouri Basin, is almost entirely dissected till plains. Soils in this area have dominantly formed in till, alluvium, or colluvium (USDA, 2006).  Corn, soybeans, sorghum, and wheat are the dominant crops, but 22% of the land is grasslands

The soil selected for the scenario is the Filbert silt loam, which is classified as a fine, smectitic, mesic Vertic Argialbolls and is in Hydrologic Group D. It is not a benchmark soil, but no benchmark soils that are used extensively for corn agriculture in C & D Hydrologic Groups occur in the county. Ten percent of the soils in Saunders County are in Group D, and Filbert soil is moderately extensive itself covering 4.5% of the land area in the county. Because D soils are somewhat extensive in the county and in order to be protective of these more vulnerable sites, a D soil was chosen to represent the county.

| Table 2. PRZM 3.12.2 Climate and Time Parameters for Saunders County, NE Corn Scenario. |  |  |
| --- | --- | --- |
| Parameter | Value | Source/Comments |
| Starting Date | Jan. 1, 1961 | Meteorological File from Omaha, NE (W94918) |
| Ending Date | Dec. 31, 1990 | Meteorological File from Omaha, NE (W94918) |
| Pan Evaporation Factor (PFAC) | 0.71 | PRZM Manual Figure 5.1 (USEPA, 2006) |
| Snowmelt Factor (SFAC) | 0.36 cm °C-1 | Maximum value of minimum range of PRZM Manual Table 5.1 (USEPA, 2006) |
| Minimum Depth of Evaporation (ANETD) | 17.5 cm | PRZM Manual (USEPA, 2006) Average of 15-20 cm |
| Pan Factor Flag (IPEIND) | 0 | Pan Factor Flag set to read from weather data as per guidance (USEPA, 2004) |

| Table 3. PRZM 3.12.2 Erosion and Landscape Parameters for Saunders County, NE Corn Scenario. |  |  |
| --- | --- | --- |
| Parameter | Value | Source/Comments |
| Method to Calculate Erosion (ERFLAG) | 4 (MUSS) | PRZM Manual (USEPA, 2006) |
| USLE K Factor (USLEK) | 0.37 tons EI-1* | USDA NRCS Soil Data Mart (http://soildatamart.nrcs.usda.gov/) |
| USLE LS Factor (USLELS) | 0.38 | Value listed for 2.6% slope at 400 feet using equation in Hann and Barfield (1978) |
| USLE P Factor (USLEP) | 0.5 | 2.6% slope row crop with contouring PRZM Manual; Table 5.6 (USEPA, 2006) |
| Field Area (AFIELD) | 172 ha | Area of Shipman Reservoir watershed (EPA, 1999) |
| NRCS Hyetograph (IREG) | 3 | PRZM Manual, Figure 5.8 (USEPA, 2006) Type II, IREG=3 |
| Slope (SLP) | 2.6% | Acreage-weighted mean value for Filbert silt loam |
| Hydraulic Length (HL) | 600 m | Shipman Reservoir (USEPA, 1999) |
| Irrigation Flag (IRFLAG) | 0 | Irrigation off |
| * EI = 100 ft-tons * in/ acre*hr |  |  |

| Table 4. PRZM 3.12.2 Crop Parameters for Saunders County, NE Corn Scenario. |  |  |
| --- | --- | --- |
| Parameter | Value | Source/Comments |
| Initial Crop (INICRP) | 1 | Set to 1 for all crops (USEPA, 2004). |
| Initial Surface Condition (ISCOND) | 1 | 1= fallow; default parameter is ignored as ERFLAG > 0. |
| Number of Different Crops (NDC) | 1 | Set to number of crops in simulation. |
| Number of Cropping Periods (NCPDS) | 30 | Set to weather data in meteorological file: Omaha, NE (W94918). |
| Maximum rainfall interception storage of crop (CINTCP) | 0.25 | Low end of corn heavy canopy; PRZM Table 5-4. (USEPA, 2006) |
| Maximum Active Root Depth (AMXDR) | 90 cm | Middle of range for corn, Table 5.9 (USEPA, 2006) |
| Maximum Canopy Coverage (COVMAX) | 100 | Set to 100 for row crops, as per guidance (USEPA, 2004) |
| Soil Surface Condition After Harvest (ICNAH) | 3 | 3 = residue, as according to guidance. (USEPA, 2004) |
| Date of Crop Emergence (EMD, EMM, IYREM) | 25/05/61 | Consistent with RUSLE dates (USDA, 2000); G87CGBSM; 15 days from planting to emergence |
| Date of Crop Maturity (MAD, MAM, IYRMAT) | 24/07/61 | 60 days after emergence |
| Date of Crop Harvest (HAD, HAM, IYRHAR) | 20/10/61 | Consistent with RUSLE dates (USDA 2000); G87CGBSM |
| Maximum Dry Weight (WFMAX) | 0.0 | Not used in scenario |
| Maximum Crop Height (HTMAX) | 300 cm | Upper end for corn from PRZM Manual Table 5.16 (USEPA, 2006) |
| SCS Curve Number (CN) | 94, 88, 94 | PRZM Table 5.10, D Soil, fallow, and row crop, contoured, poor condition (USEPA, 2006) |
| Manning’s N Value (MNGN) | 0.014 | RUSLE Project; G87CGBSM; corn for grain, Lincoln weather station; mulch tillage (USDA, 2000). |
| USLE C Factor (USLEC) | 0.039-0.501 | RUSLE Project; G87CGBSM; corn for grain, Lincoln weather station; mulch tillage (USDA, 2000). |

| Table 5. PRZM 3.12 Filbert silt loam Soil Parameters for Saunders County, NE. |  |  |
| --- | --- | --- |
| Parameter | Value | Source/Comments |
| Total Soil Depth (CORED) | 158 cm | NRCS Soil Data Mart (SDM) (http://soildatamart.nrcs.usda.gov) |
| Number of Horizons (NHORIZ) | 4 | NRCS Soil Data Mart (SDM) |
| Horizon Thickness (THKNS) | 10 cm (HORIZN = 1) 8 cm (HORIZN = 2) 20 cm (HORIZN = 3) 120 cm (HORIZN = 4) | NRCS Soil Data Mart (SDM). The top horizon was split into two horizons as per PRZM Scenario Guidance (USEPA, 2004). |
| Bulk Density (BD) | 1.3 g/cm3 (HORIZN = 1) 1.3 g/cm3 (HORIZN = 2) 1.3 g/cm3 (HORIZN = 3) 1.15 g/cm3 (HORIZN = 4) | NRCS Soil Data Mart (SDM) (http://soildatamart.nrcs.usda.gov). Midpoint of the reported range; PRZM Scenario Guidance (EPA, 2004). |
| Initial Water Content (THETO) | 0.386 cm3/cm3 (HORIZN =1) 0.386 cm3/cm3 (HORIZN =2) 0.345 cm3/cm3 (HORIZN = 3) 0.35 cm3/cm3 (HORIZN = 4) | NRCS Soil Data Mart (SDM); values are mean available water plus the wilting point water content of Filbert silt loam soils. |
| Compartment Thickness (DPN) | 0.1 cm (HORIZN = 1) 2 cm (HORIZN = 2) 4 cm (HORIZN = 3) 4 cm (HORIZN = 4) | NRCS Soil Data Mart (SDM) (http://soildatamart.nrcs.usda.gov); PRZM Scenario Guidance (USEPA, 2004). |
| Field Capacity (THEFC) | 0.386 cm3/cm3 (HORIZN =1) 0.386 cm3/cm3 (HORIZN =2) 0.345 cm3/cm3 (HORIZN = 3) 0.35 cm3/cm3 (HORIZN = 4) | NRCS Soil Data Mart (SDM); values are mean available water plus the wilting point water content of Filbert silt loam soils. |
| Wilting Point (THEWP) | 0.156 cm3/cm3 (HORIZN =1) 0.156 cm3/cm3 (HORIZN =2) 0.115 cm3/cm3 (HORIZN = 3) 0.230 cm3/cm3 (HORIZN = 4) | Estimated using Table 5.23 in PRZM 3.2 manual with NRCS Soil Data Mart Data for Filbert silt loam soil. |
| Organic Carbon Content (OC) | 1.740% (HORIZN = 1) 1.740% (HORIZN = 2) 0.435% (HORIZN = 3) 0.435% (HORIZN = 4) | NRCS SDM; values for horizons 1 to 3 = mean %OM / 1.724. PRZM Scenario Guidance (USEPA, 2004). |

References

Haan, C.T., and B.J. Barfield.  1978.  Hydrology and Sedimentology of Surface Mined Lands.  Office of Continuing Education and Extension, College of Engineering, University of Kentucky, Lexington KY 40506.  pp 286.

USDA.  2000.  Revised Universal Soil Loss Equation (RUSLE) EPA Pesticide Project.  U.S. Department of Agriculture, National Resources Conservation Service (NRCS) and Agricultural Research Service (ARS).

USDA.  2005.  2002 Census of Agriculture.  U.S. Department of Agriculture, National Agricultural Statistics Service (NASS).  Online at: http://www.nass.usda.gov/index.asp.

USDA.  2003.  Official Series Description – Filbert Series.  U.S. Department of Agriculture, Natural Resources Conservation Service (NRCS).  Dec. 2006.  Online at: http://ortho.ftw.nrcs.usda.gov/osd/dat/F/FILBERT.html

USEPA.  1999.  Jones, R.D., J. Breithaupt, J. Carleton, L. Libelo, J. Lin, R. Matzner, and R. Parker. Guidance for Use of the Index Reservoir in Drinking Water Exposure Assessments. Environmental Fate and Effects Division, Office of Pesticide Programs, U.S. Environmental Protection Agency, Washington, DC.

USEPA.  2004.  Abel, S.A.  Procedure for Conducting Quality Assurance and Quality Control of Existing and New PRZM Field and Orchard Crop Standard Scenarios.  Environmental Fate and Effects Division, Office of Pesticide Programs, U.S. Environmental Protection Agency.

USEPA.  2006.  Carsel, R.F., J.C. Imhoff, P.R. Hummel, J.M. Cheplick, and A.S. Donigian, Jr.  PRZM-3, A Model for Predicting Pesticide and Nitrogen Fate in the Crop Root and Unsaturated Soil Zones: Users Manual for Release 3.12.2.  National Exposure Research Laboratory, Office of Research and Development, U.S. Environmental Protection Agency, Athens, GA.