More Critical Look for Potential Errors Requiring Attention in New Jersey Melon Scenario

ü-> Input file has the word "Regional" in the file name.

ü-> Table titles indicate PRZM version 3.12; web site indicates version 3.12.2 is the currently approved version.

ü-> MLRA in input file. MRLA in metadata file.  One is transposed.  S149a "a" is lower case in metadata file and upper case in input file.

ü-> It seems that a "nearly level coastal plain..." would not have slopes ranging as high as 40%?   Bet all the local kids know where that sledding hill is!

This also means I've no idea how to interpret the SLP of 3.8%.  The other scenarios used the average of the reported range. (deleted 40% sentence)

ü-> SFAC, CINTCP lack units in metadata file.

ü-> AMXDR comments indicate the root depth came from information regarding an irrigated crop.  With no irrigation here, is that still the same?

If it is an issue, I didn't notice it in the other 2 melon scenarios so it wasn't noted.  If not an issue, even better and ignore the comment. (not an issue)

ü-> COVMAX lacks % sign in metadata file.

ü-> MNGN, USLEC comments reference strawberries and York.  Perhaps add a disclaimer indicating closest crop practice, etc. in RUSLE PRoject.

ü-> THETO and THEFC metadata file comments for other melon scenarios were "mean available water plus the wilting point water content of ... [name the soil]"

This scenario comment is "mean 1/3 - bar water contents of ... [name the soil]".  Which comment is correct or are both correct?  (They’re the same, no issue)

ü-> DPN metadata file comment must be in the wrong place??  How could a thickness be a water content value?  Perhaps related to the previous potential error. (comment corrected)

ü-> Second reference URL goes to muskmelons in Maryland rather than Melons in California.  What is the correct URL? (reference is fixed)

ü-> KDFLAG and adsoprtion questions for the input file as in the other melon scenarios.  I still don't know if I should be looking for it in Record 37 (it isn't there but KDFLAG comments say if KDFLAG=0, it should be) or maybe this version of the model doesn't use it? (not an issue)

ü-> THKNS input file comments regarding balance of 36 cm and 100 cm horizon still confuse me.  Even more this time since I noticed that the CORED is quite a bit less than 100 cm.  What are these 36 and 100 numbers indicating? (comments removed from input file)