TO:	Mark Corbin, US EPA OPP/EFED
Milwina Locklear, US EPA OPP/EFED

FROM:	Tim Negley, SRC, negley@syrres.com, 315.452.8260.

CC:	Gail Maske-Love, US EPA OPP/EFED

DATE:	1/19/2007

SUBJECT:	TD4 Revised Deliverable and Response to WQTT review: Outdoor Ornamental Nursery Scenarios.  (GSA Contract No. GS-00F-0019L, Order Number. EP06H000149)

Mark,

Please find attached the revised excel files and corresponding metadata for the TN scenarios. SRC reviewed the comments from the WQTT and has addressed them as detailed in attachment A of this memo.

Sincerely,

Tim Negley

ATTACHMENT A

Ornamental Nursery Scenario Specific and Global Comments/Revisions

Specific Responses to Reviewer Comments

| MI Nursery |  |
| --- | --- |
| CN & Site Selection | See Global Changes/Comments |
| Review requested additional soil description in excel file. | Completed |
| S. California Nursery |  |
| USLEK | Changed USLEK to 0.32 consistent with Cieneba course sandy loam. |
| HL | Retained HL as per PRZM guidance (EPA, 2004) |
| IRTYP | Changed IRTYP to 4 (drip irrigation). |
| FLEACH | Changed FLEACH to 0 as per guidance for drip irrigation (EPA, 2005) |
| RATEAP | Changed RATEAP to 0.046 as per guidance and CN = 82 |
| ICNAH / ISCOND | Change ICNAH and ISCOND to 1 for fallow conditions as recommended. |
|  | Crop parameters are based on J. Bethke as per guidance and EFED direction on 1/9/2007. |
| CN | Changed Third CN to 87 to represent gravel road if nursery is cleared as per reviewer suggestion. This triggered changing the CN in record 9E as well. |
| Record 9E | Changed record 9E curve number to represent a cleared condition after harvest and between emergence. |
| RUSLE C Factors | See Global Changes/Comments |
| Other | Added citation for USGS GIRAS data. |
| Oregon Nursery |  |
| CN & Site Selection | See Global Changes/Comments |
| AFIELD | A field was not changed, but kept at 172 as per EFED guidance (EPA, 2004) |
| COVMAX/HTMAX | See Global Changes/Comments for crop cover parameters |
| Tennessee Nursery |  |
| CN & Site Selection | See Global Changes/Comments |
| Crop parameters to generic | See Global Changes/Comments for crop cover parameters |
| CN |  |
| Metstation | Added location description to metadata as requested. |
| Reviewer indicated that a regional assessment (including soils) could be performed by using nursery addresses | EFED indicated this is not necessary (see 1/9/2007 telecon. notes). The scenario represents conservative soils in the primary nursery county, and the soil selected is based on information from local extension agent. No additional work on this is required at this time. |
| Regional assessment | No performed as directed. See global comments for scenario location/vulnerability. |
| pH | Added description of pH. |
| Florida Nursery |  |
| No comments | See Global Changes/Comments |
| NJ Nursery |  |
| Location Justification | See Global Changes/Comments |
| File name | Changed “Nursery” to “nursery”. |
| Other | Corrected spelling of “moderately” in paragraph 4. |
| Other | Corrected incorrect reference to Captina in paragraph 5. |
| USLELS | Changed from 0.5 to 0.37 according to Haan and Barfield, 1978). |
| IRTYP, FLEACH, PCDEPL, RATEAP | Added documentation to the metadata as requested. |

Global changes/comments

Scenario Location/Vulnerability: Several reviewers expressed concern over the selection of the six scenarios with regard to regional vulnerability.  In an email (12/22/2006) MC pointed out from the SOW, SRC was not tasked to locate the regional scenarios; rather the SOW specified which scenarios to develop and where to focus them.  Although EFED located the scenarios based on nursery hotspots (peak acreages), a regional vulnerability assessment would be useful.  This is routinely problematic in scenario development.  At this time, MC directed SRC not to perform a regional vulnerability assessment.  (see telcon notes from 1/9/2007)

RUSLE Factors: Added note to the RUSLE parameters indicating the new method for associating RUSLE dates and C factors with emergence dates. “Note: RUSLE dates and C factors from the RUSLE project have been reordered and tied to emergence dates.”

Excel file comments: Reviewers indicated there should be more documentation in the excel files.  In a teleconference MC indicated that this comment is not applicable (see telcon notes from 1/9/2007).  All documentation should be included in the corresponding metadata file, not the excel file.

Curve number:

EFED SRC discussed CN selection during a teleconference on 1/9/2007.  SRC provided a distribution of CNs after the call to put the nursery CNs in perspective.  EFED reviewed and found no reason to change the CNs from the farmstead value used.  EFED requested more documentation and potentially the appropriate CN figure be added to the metadata.  SRC did both.

Crop/Cover Parameters:  A general statement was added to the metadata that nursery scenarios are diverse and parameters have been chosen to be representative of nurseries in general.

References Cited

EPA.  2004.  Abel, S.A.  Procedure for Conducting Quality Assurance and Quality Control of Existing and New PRZM Field and Orchard Crop Standard Scenarios.  Environmental Fate and Effects Division, Office of Pesticide Programs, U.S. Environmental Protection Agency, Washington, DC. Revised 2004.

Haan, C.T. and B.J. Barfield.  1978.  Hydrology and Sedimentology of Surface Mined Lands.  Office of Continuing Education and Extension, College of Engineering, University of Kentucky, Lexington KY 40506.  pp 286.