More Critical Review for Potential Errors Requiring Attention in Minnesota Corn Scenario

-> Table titles in the metadata file indicate version 3.12; web site references version 3.12.2 as the current approved version.

-> MLRA in input file; MRLA in metadata file.  One is transposed.  103 in input file; M103 in metadata file.

-> SFAC, CINTCP lack units in metadata file.

-> COVMAX lacks % sign in metadata file.

-> EMM, MAM in the input file use single digits rather than the leading zeroes used in the metadata file. (input file adjusted; does not affect output)

-> HTMAX input file comments reference NC Corn scenario while HTMAX metadata file comments reference PRZM manual. (not an issue

-> MNGN, USLEC: that 16 set number for any row (that Dave told me to watch for) is exceeded in the input file with 18 sets for the first row. (fixed, did not affect output)

-> Horizon 2 HORIZN comment in input file: "bottom 26 cm..." yet THKNS is 30 cm? (deleted comment)

-> Horizons 4 and 5 BD comment in input file:  "original value 1.85 causes PRZM to crash..." yet no explanation for how substitute value of 1.58 was selected. (deleted comment)